CV5570 – Computer Vision for Construction Engineering

Object Detection

Assignment 02

By

Vasantharan K

ID26S012

Date: 02.10.2026

Assignment 2

CV5570 – Computer Vision for Construction Engineering

Objective

The objective of this assignment is to develop and evaluate an object detection pipeline for construction equipment using three different detector families: a two-stage detector, a one-stage detector, and a transformer-based detector.

The Annotated Construction Images Dataset (ACID) is used for the experiments. ACID was developed specifically for construction-machine detection and contains images of construction equipment with manually annotated object locations. The original ACID study reported 10,000 images covering 10 construction-machine categories and evaluated several object detection algorithms including YOLOv3, Inception-SSD, R-FCN-ResNet101, and Faster R-CNN-ResNet101.

For this assignment, the experiments are restricted to three classes:

- Excavator

- Dozer

- Dump Truck

The following three detectors are implemented:

- Two-stage: Faster R-CNN with ResNet-50-FPN

- One-stage: YOLOv8s

- Transformer-based: DETR with ResNet-50

The models are compared using detection accuracy, per-class average precision, inference speed, parameter count, training time, convergence behaviour, and qualitative success and failure cases.

The main objective is not only to determine which detector achieves the highest accuracy, but also to understand the practical trade-off between accuracy, computational cost, inference speed, and failure behaviour for construction-site monitoring.

Images/Dataset Description, Acquisition and Preprocessing

ACID Dataset

The ACID dataset is a construction-machine image dataset developed for deep-learning-based object detection. The dataset provides bounding-box annotations for construction machinery and was designed to support applications such as construction productivity monitoring and safety monitoring. The official ACID website states that annotations follow consistency, correctness, and completion standards, including annotation of visible and occluded construction machines.

For this assignment, a three-class subset was extracted containing 3,929 images and 7,614 annotated object instances.

The annotation data were available in both CSV and JSON formats. The CSV file was used as the primary annotation source during preprocessing because the bounding-box coordinates were directly available in tabular form.

The CSV annotation contains the following information: ”Image filename, Image width, Image height, Object class, xmin, ymin, xmax and ymax.”

The JSON annotation follows a structured image-category-annotation organization.

Dataset Statistics

| Class | Images Containing Class | Instances |
| --- | --- | --- |
| Excavator | 2,239 | 2,790 |
| Dozer | 1,025 | 1,123 |
| Dump Truck | 2,206 | 3,701 |
| Total | 3,929 unique images | 7,614 |

The dataset therefore has a moderate class imbalance. Dump Truck is the most frequent class with 3,701 instances, while Dozer has the fewest instances with 1,123.

The class imbalance is important when interpreting the detector results because a model can achieve good overall performance while performing substantially worse on the less frequent Dozer class.

Bounding Box Analysis

For every annotation, the following quantities were calculated:

- Bounding-box width

- Bounding-box height

- Bounding-box area

- Bounding-box aspect ratio

- Bounding-box area relative to image area

The bounding-box width ranged from 12 to 3,768 pixels, while the height ranged from 10 to 2,696 pixels.

The mean bounding-box width was approximately 391.45 pixels, and the mean height was approximately 271.64 pixels.

The mean bounding-box aspect ratio was 1.51, with values ranging from 0.164 to 20.90.

The mean bounding-box area ratio was approximately 0.168, indicating considerable variation in the relative size of construction equipment within images.

These variations make the dataset suitable for comparing detectors under different object scales and aspect ratios.

| Fig 1. Class frequency distribution | Fig 2. Bounding-box width distribution |
| --- | --- |
| Fig 3. Bounding-box height distribution | Fig 4. Bounding-box aspect ratio distribution |
| Fig 5. Relative bounding-box area distribution |  |

Fig 6. Six sample images with ground-truth bounding boxes

Train/Validation/Test Split

The dataset was divided at the image level into training, validation, and test sets using a 70:15:15 ratio.

A fixed random seed of 42 was used to make the split reproducible. Splitting was performed at the image level rather than at the bounding-box level so that annotations from the same image could not appear in different subsets.

The resulting split was:

| Split | Images |
| --- | --- |
| Training | 2,750 |
| Validation | 589 |
| Test | 590 |
| Total | 3,929 |

No overlap was found between the three image sets.

The number of annotated instances in each split was:

| Split | Excavator | Dozer | Dump Truck |
| --- | --- | --- | --- |
| Train | 1,943 | 806 | 2,618 |
| Validation | 418 | 161 | 528 |
| Test | 429 | 156 | 555 |

The same image-level split was used across the detector families to maintain consistency in evaluation.

Two – Stage Object Detection

Model Configuration

The model used was:

Faster R-CNN + ResNet-50-FPN

The model was initialized with COCO-pretrained weights and fine-tuned for the three ACID classes.

Training Configuration

| Parameter | Configuration |
| --- | --- |
| Model | Faster R-CNN ResNet-50-FPN |
| Pretrained weights | COCO |
| Classes | 3 |
| Batch size | 2 |
| Epochs | 10 |
| Optimizer | SGD |
| Learning rate | 0.005 |
| Momentum | 0.9 |
| Weight decay | 0.0005 |
| LR scheduler | StepLR |
| Step size | 3 epochs |
| LR decay | 0.1 |
| Hardware | NVIDIA GeForce RTX 4070 Laptop GPU |
| Training time | 95.01 min |

No additional image augmentation was introduced.

Training and Validation Loss

The Faster R-CNN training loss decrease d consistently during training.

| Epoch | Training Loss | Validation Loss |
| --- | --- | --- |
| 1 | 0.2512 | 0.1938 |
| 2 | 0.1709 | 0.1730 |
| 3 | 0.1406 | 0.1634 |
| 4 | 0.0980 | 0.1536 |
| 5 | 0.0886 | 0.1516 |
| 6 | 0.0833 | 0.1552 |
| 7 | 0.0777 | 0.1564 |
| 8 | 0.0770 | 0.1566 |
| 9 | 0.0767 | 0.1570 |
| 10 | 0.0761 | 0.1571 |

The training loss continued to decrease throughout the ten epochs. The validation loss decreased rapidly during the first five epochs and then remained approximately stable. The small increase after epoch 5 indicates that further optimization primarily improved training loss without producing a corresponding improvement in validation loss.

Fig 7. Faster R-CNN training and validation loss curve

Faster R-CNN Quantitative Results

The validation results were:

| Metric | Result |
| --- | --- |
| mAP@0.5 | 0.9203 |
| mAP@0.5:0.95 | 0.6931 |

The per-class AP values reported by the evaluation were:

| Class | AP@0.5:0.95 |
| --- | --- |
| Excavator | 0.7184 |
| Dozer | 0.7382 |
| Dump Truck | 0.6227 |

The Dump Truck class has the lowest AP among the three classes despite having the largest number of training instances. This suggests that class frequency alone does not determine detection performance and that factors such as appearance variation, object scale, occlusion, and localization difficulty can also affect performance.

Faster R-CNN Qualitative Results

Six qualitative examples were generated:

- Three successful detections

- Three failure cases

One – Stage Object Detection

YOLO

Transformer – Based Object Detection

Experimental Comparison

Limitations

Conclusion

References

Coding References

GitHub Link: https://github.com/vasantharan/CV5570_Assignment_2**

**Repo will be made public after submission deadline

-  END -